Part 1 REQUEST FOR SCAFFOLDING

I hereby request that the scaffolding for purpose of …………………….. be erected for the duty (select from load table as given below) and height………..Meter.

| Load duty | Light |  | Medium | Heavy/Special |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Safe load in Kg | 75 | 150 | 225 | 300 | 450 | 600 |
| Maximum bay length / width in Meter | 2.7 | 2.2 | 1.8 | 1.4 | 1.0 | 0.8 |
| Tick whichever applicable |  |  |  |  |  |  |

(Name & Signature of Maintenance Engineer)

M/s ________________				Date:

(Scaffolding Contractor)

Part 2 INSPECTION CHECK LIST

| S. No | Checks Points | Yes/ No/ NA | Remarks |
| --- | --- | --- | --- |
|  | Are the scaffoldings erected as per the load duty requested? |  |  |
|  | Is every work platform fitted with handrail (top rail 1200 mm and mid rail at 600 mm high) and a toe board (150 mm) secured to the platform sides? |  |  |
|  | Are the scaffolding in plumb and level? |  |  |
|  | Are the planks/gratings placed in order without undue gaps and anchored? |  |  |
|  | Are the footing / anchorage for scaffolds sound and the bay lengths maintained as per the maximum Intended load (duty)? |  |  |
|  | Are the poles, legs or uprights of scaffolding are securely braced to prevent swaying / displacement? |  |  |
|  | Are the base plates (100 x 100 x 3 mm) provided for scaffolding posts? |  |  |
|  | Are the base plates are supported by sole plate (300 x 300 x 6 mm) at unpaved area? |  |  |
|  | Is kicker lift (at distance 150 mm from ground/base) provided for scaffold of height more than 6.0 meter? |  |  |
|  | If the scaffolding is erected above walkways or work areas, are the space between toe boards and railings screened? |  |  |
|  | If work is done over men who are working on scaffolding, is overhead protection provided? |  |  |
|  | Has the scaffolding area suitably barricaded during erection / usage / dismantling? As far as practicable, barricades shall be provided by keeping a clear distance of 1.0 meter from scaffold structure. |  |  |
|  | Is free standing scaffolding tower protected from tipping by guying or other means? |  |  |
|  | Is there a safe and convenient means of access? If a ladder is used, is it rising at least 1050mm above the platform? |  |  |
|  | Are relevant status boards (tags) attached to the scaffolding completed / under erection as applicable? |  |  |
|  | Is safe access to equipment or emergency egress restricted by scaffolds? |  |  |
|  | Are wall scaffolding anchored every 10 m of length and 8 m of height? |  |  |
|  | Are landing platforms provided for access ladders more than 9 meters height? |  |  |
|  | If two pipes are connected, are they overlapped at least 600 mm and at least two clamps used? |  |  |

Part 3 CERTIFICATES OF FITNESS OF SCAFFOLDING

I hereby certify that the scaffolding erected for work permit no.  ------------------- is ready and fit for use. I further certify that it is intended for light duty/ General purpose (Medium duty)/ Heavy duty or special duty (tick whatever applicable) service and fit for a load of …………….. Kg and should not be overloaded.

Name and signature of                                                  Name and signature of

Maintenance Engineer                            		        Scaffolding Supervisor

(Dept……………..)				        M/s _________________

(Scaffolding Contractor)

Date:

Part 4 WEEKLY RE-CERTIFICATION FOR FITNESS OF SCAFFOLDING

| Certification for | Date | Scaffolding is inspected as per part 2 (any new observation) | Scaffolding is safe for use ( Yes/ No) | Name and sign of scaffolding supervisor | Name & sign of maintenance engineer |
| --- | --- | --- | --- | --- | --- |
| 2nd week |  |  |  |  |  |
| 3rd week |  |  |  |  |  |
| 4th week |  |  |  |  |  |
| 5th week |  |  |  |  |  |
| 6th week |  |  |  |  |  |
| 7th week |  |  |  |  |  |
| 8th week |  |  |  |  |  |
| 9th week |  |  |  |  |  |
| 10th week |  |  |  |  |  |

Annexure 1: Safety Boards

SCAFFOLDING INCOMPLETE DO NOT USE

Scaffold No SCAFFOLDING COMPLETE READY FOR USE Name of Contractor :

Annexure 2: Scaffolding Audit check list

| S. No | Checks Points | Rating | Remarks |
| --- | --- | --- | --- |
|  | Has the request for scaffolding been signed by authorized person? |  |  |
|  | Has the 'certificate of fitness of scaffolding' been signed by a scaffolding supervisor and maintenance engineer? |  |  |
|  | Are the scaffoldings erected as per the load duty requested? |  |  |
|  | Is every work platform fitted with handrail (top rail 1200mm and mid rail at 600 mm high) and a toe board (150mm) secured to the platform sides? |  |  |
|  | Are the scaffolding in plumb and level? |  |  |
|  | Are the gratings placed in order without undue gaps and anchored? |  |  |
|  | Are the footing / anchorage for scaffolds sound and the bay lengths maintained as per the maximum intended load? |  |  |
|  | Are the poles, legs or uprights of scaffolding are securely braced to prevent swaying / displacement? |  |  |
|  | Are the base plates (100 x 100 x 3 mm) provided for scaffolding posts? |  |  |
|  | Are the base plates are supported by sole plate (300 x 300 x 6 mm) at unpaved area? |  |  |
|  | Is kicker lift (at distance 150mm from ground/base) provided for scaffold of height more than 6.0 meter? |  |  |
|  | If the scaffolding is erected above walkways or work areas, are the space between toe boards and railings screened? |  |  |
|  | If work is done over men who are working on scaffolding, is overhead protection provided? |  |  |
|  | Has the scaffolding area suitably barricaded during erection / usage / dismantling? As far as practicable, barricades shall be provided by keeping a clear distance of 1.0 meter from scaffold structure. |  |  |
|  | Is free standing scaffolding tower protected from tipping by guying or other means? |  |  |
|  | Is there a safe and convenient means of access? If a ladder is used, is it rising at least 1050mm above the platform |  |  |
|  | Are relevant status boards (tags) attached to the scaffolding completed / under erection as applicable? |  |  |
|  | Is safe access to equipment or emergency egress restricted by scaffolds? |  |  |
|  | Are wall scaffolding anchored every 10 m of length and 8 m of height? |  |  |
|  | Are landing platforms provided for access ladders more than 9 meters height? |  |  |
|  | If two pipes are connected, Are they overlapped at least 600 mm and at least two clamps are used? |  |  |
| Total Marks scored |  |  |  |
| Overall Rating |  |  |  |

Audited By: 									Date:

Auditor 1: Name:  ____________________Sign:___________	_________

Auditor 2: Name:  ____________________Sign:___________

Auditor 3: Name:  ____________________Sign:___________

Annexure 3: Scaffold pipe, Ladder, Plank/Jalli, Clamps, Base Plate Check-list

| Material | Check List | Observation |
| --- | --- | --- |
| ###### Pipes | - Rust Free Pipes - No Bends, Cuts, Holes - No Sharp edge, pipe end condition - Pipe thickness minimum 2 mm - Painted / galvanized pipes - Any other(Specify) |  |
| ###### Ladder | - Rail Damaged (cracks, deformation etc.) - Rust free & Properly Painted Ladder. - No Cut, Hole or any Physical damage. - Equal spacing between rungs (max 30 cm). - Ladder width greater than = 30 cm. - No Bent, Broken, loose Rungs. - No Sharp edge. - No Oil, Grease and other slippery items - Bottom non-skid pad damaged/missing - Top and bottom rung tie rod provided - Any other(Specify) |  |
| ###### Plank/Jalli | - Jalli spacing maximum 1.5” to 2”. - Plank all 4 hooks not damaged. - No Cut or any other Physical damage. - Not depressed beyond 15 mm. - Rust free and properly painted. - Flat Thickness 3mm internal and 5mm outer boundary. - Any other(Specify) |  |
| ###### Clamps | - Rust free and properly painted. - No cut/cracks or physical damage. - No lubrication. - No Eyebolts bent. - Thread condition/Nut condition. - No welding / local repair - Any other(Specify) |  |
| Base Plate & Sole plate | - Rust free and properly painted. - No bend. - No Cut, Hole or physical damage. - Flat Bottom portion. - Any other(Specify) |  |

Inspected By: Name:  _______Designation:_____	Sign: ______Date:_____

Reviewed By: Name:  _______Designation:_____	Sign: ______Date:_____

Annexure 4: Do’s and Don’ts for Scaffold Safety

Do’s

- Use only certified scaffolding, tagged with “Scaffolding Complete, Ready for Use”

- Maintain three-point-contact while climbing up or down the ladder.

- Use rope or container or other suitable means for carrying material from or to ground.

- Maintain good housekeeping, especially at elevated working platform and near the access.

- Do take special precautions while working in vicinity of mobile cranes or electrical overhead lines.

- Wear full body safety harness with double lanyard/ personal fall arrest system or other equally effective means while ascending or descending the ladder or scaffolding working platform is not fully guarded.

- Do check for emergency approach/ evacuation, especially adequacy means of egress.

- Use safe material handling ways while shifting the material.

- Do report “At-risk conditions” e.g. missing ladder, incomplete decking, improper guarding, no landing platform, loosely clamped ladder, etc. to your supervisor.

- Do check weekly certification of scaffold at random

Don’ts

- Do not use any part of operational line or pressurized piping as a support, or tie point for scaffolds or staging.

Do not permit mixing of scaffold types in any one structure (e.g., steel/aluminum couplers, wooden boards/metal grid boards).

- During the possibility of a thunderstorm, no work shall be executed at a height where a person can be exposed to lightning.

- Never keep loose tools, materials etc at height in a way it may roll off and fall from height.

- Do not use cross-bracing or end-frame (unless specially designed) for access.

- Avoid overloading and impact loading

- Never alter or modify or dismantle any part of the scaffold without permission from scaffolding supervisor or Tata Power personnel.

- Do not throw tools or scaffolding materials from height.

- Do not use “Incomplete Scaffold”.