|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Working in Man lift (Aerial Lift) |  |  |
|  | Task | Hazards | Controls |
|  | - Prepare work area. | Injuries to operator, co-workers, and bystanders | Warn people in work area; utilize barricades, cones, or caution tape to keep bystanders a safe distance from job; position lift away from traffic, building overhangs, and power lines; level truck base and block wheels. Visual inspection and operational check of aerial lift |
|  | 2. Operate lift. | Falling injuries | Secure operator with a body belt and personal lanyard anchored to boom. Never exceed the posted lift capacity (including worker, material, and tools). Never use lift as a crane or hoist to lift or lower materials. Always look in the direction of travel of the bucket when operating lift; never run boom or bucket into conductors, cables, poles, trees, etc. Never move the lift truck when the boom is elevated in a working position with an operator in the bucket. No part of the body shall be used to locate or to attempt to stop a hydraulic leak. |
|  |  | Eye, hand, arm injuries to operator | Wear hard hat, gloves, and safety glasses. |
|  |  | Electrical injuries to operator. | Maintain a minimum clearance of 10 feet from energized conductors rated 50kV phase-to-phase or less; for lines rated over 50kV phase-to-phase, the minimum clearance shall be 10 feet plus 4/10 inch for each kilovolt greater than 50 kV phase-to-phase. WARNING: Insulated buckets do not protect operators from other electrical paths to the ground, such as those through trees, through a guy wire, or the path from one phase wire to the second phase wire, any one of which can be fatal. |
|  | Required Training: - Operation of aerial lift - Electrical safety - Fall protection | Required Personal Protective Equipment (PPE) - Hard hat - Gloves - Safety glasses - Body belt - Lanyard |  |