|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Using an Angle Grinder |  |  |
|  | Task | Hazards | Controls |
|  | - Check cord integrity. | Hand cut from cut wires. | Wear leather gloves. Inspect slowly. |
|  | 2. Check conditions of grinding wheel and appropriate RPM. | (None foreseen) |  |
|  | 3. Check grinding wheel tightness. | Hand injury from inadvertent starting | Do not plug in the machine until inspection is complete. |
|  | 4. Verify the guard is tight and appropriate for the job. | Foot injury from dropping the tool | Rest the tool on the bench. Wear steel-toed shoes. |
|  | 5. Verify the appropriate handle location. | Foot injury from dropping the tool | (See controls for Task 4.) |
|  | 6. Inspect trigger for physical damage and proper operation. | (None foreseen) |  |
|  | 7. Make sure the materials being ground are adequately secured and positioned correctly. | Injuries associated with the work propelled by the grinder and/or landing on you | Verify the work is adequately secured by trying to dislodge it with a gloved hand (the work weight may secure it enough). Wear steel-toed shoes. |
|  | 8. Plug-in the grinder. | Eye and skin damage from projectiles. | Check the trigger switch to insure it is off. |
|  | 9. Begin grinding. | Eye injuries from projectiles and sparks | Wear safety glasses/goggles and a face shield. |
|  |  | Skin damage from sparks and projectiles | Wear leather gloves, long sleeved shirt, long pants, or leather welding guards. |
|  |  | Hearing loss | Wear ear plugs. |
|  |  | Ergonomic considerations. | Change position from time to time. Wear vibration resistant gloves. |
|  |  | Inhalation of toxic or irritant fume or particulate | Wear the appropriate respirator based on the content of the metal and its coatings. Contact EH&S (2-3073) for evaluation and exposure assessment. Use local or dilution ventilation to direct or collect fumes and/or particulate |
|  | Required Training: - Operation of the angle grinder - Hearing protection - Eye protection | Required Personal Protective Equipment (PPE) - Leather gloves - Eye and face protection - Body covering - Foot protection |  |