|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: | Task: |  | Portable band saws |
| --- | --- | --- | --- |
|  | Name of Shop or Dept: |  | OCH Maintenance, UA |
|  | Job Title(s): |  | Building Maintenance, Plumbers |
|  | Analyzed by: |  | M. Williams |
|  | Date: |  | 05/01/13 |
| Required PPE: |  |  |  |
| Work boots hearing protection safety glasses Gloves |  |  |  |
| Required/Recommended Trainings: |  |  |  |
| STF Safety Equipment safety precautions |  |  |  |
| TASK | HAZARDS | CONTROLS |  |
| Operating a portable band saw | Lacerations Amputations Struck By’s Slips Trips Falls | - Don PPE, including safety glasses, work boots, gloves and hearing protection (When working in noisy areas). - Always unplug or remove battery pack before removing blades or accessories. - Do not wear loose clothing. Keep jewelry and hair out of reach of moving parts. - Before use check for damage to blade and/or teeth. Cracked and damaged blades can vibrate loose and become projectiles. - When cutting do not hold piping below or in line of blade. - Always cut away from the body. - Never force the tool as it can bind and break resulting in flying objects. - Extended use can create vibration; gloves will help reduce vibration to hand and arms. Take breaks when working on long projects. - Do not touch blade immediately after use as it will be hot. - Never attempt to cut through piping that may be insulated with possible asbestos containing Material (PACM), only those trained in asbestos should work around PACM or ACM. |  |