|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Operating an Air Blower |  |  |
|  | Task | Hazards | Controls |
|  | - Check/add fuel. | Pinch points for fingers, vision damage | Wear gloves and other safety items; avoid pinch points by keeping eyes on task |
|  |  | Skin contact with substances from spills or splashes, inhalation of vapors | Add gasoline and additives in safe, uncrowded, and well-ventilated area; wear eye protection; clean up small fuel spills (<1 gallon) and medium spills (1 – 5 gallons) immediately; in case of large spills (>5 gallons), contact EH&S. |
|  | 2. Start blower. | Muscle strain | Pull cord straight back from blower in one smooth motion; check line of fire of arm and elbow for obstacles. |
|  |  | Hearing damage to self and others | Wear ear plugs. |
|  |  | Dust inhalation by self and others | Wear dust mask; pause for pedestrians or motorists to move past immediate location. |
|  | 3. Use blower to clean area. | Muscle strain | Use easy, short sweeping motion; avoid twisting and bending. |
|  |  | Hearing damage to self and others | Wear ear plugs. |
|  |  | Dust inhalation by self and others | Wear dust mask; pause for pedestrians or motorists to move past immediate location. |
|  | Required Training: - Operation of blower - Knowledge of City of Berkeley blower ordinance - Spill response | Required Personal Protective Equipment (PPE) - Safety glasses - Gloves - Ear plugs - Dust mask |  |