|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Operating a Chainsaw |  |  |
|  | Task | Hazards | Controls |
|  | - Prepare work area. | Tripping hazards | Clear brush and debris, visually examine area to establish secure footing. |
|  |  | Injuries to bystanders and co-workers | Warn people in or near area; utilize barricades, cones, or caution tape to keep bystanders a safe distance from job; consider the shape and lean of tree; consider wind force; plan escape route from work area in case of danger. |
|  | 2. Start saw | Cutting injuries to hands, feet, body parts | Start saw on ground with foot on rear handle, left hand firmly gripping saw grip, right hand pulling starter cord. Maintain secure footing. Wear gloves, hard hat, chaps, sturdy work boots. Saws weighing less than 15 pounds may be drop started. Make sure co-workers are clear of saw. |
|  |  | Flying debris entering eyes | Wear safety glasses with side protection, face shield. |
|  |  | Hearing loss | Wear ear plugs or ear muffs. |
|  | 3. Cutting wood | Cutting injuries to hands, feet, body parts | Maintain proper stance and hold; keep left arm and elbow straight, two hands on saw, body to the left of saw; maintain secure footing; never cut above shoulder level; cut with the underside of saw blade as much as possible. Stop engine for all cleaning, refueling, adjustments, and repair of saw. |
|  |  | Log or limb rolling into body | Stand upslope so cut pieces roll away from feet, legs, and body. |
|  |  | Kickback, blade bucking away from wood | Make sure blade is moving before making contact with material; use wedge when necessary to keep blade from binding; make sure solid objects like rocks or concrete do not contact blade; use 90-degree notch and backcut techniques on standing trees >5 inches in diameter. |
|  |  | Flying debris entering eyes | Wear safety glasses with side protection, face shield. |
|  |  | Muscle strain, fatigue | Utilize proper body position; stretch, take frequent breaks to maintain alertness; stay hydrated. |
|  |  | Injuries to bystanders and co-workers | Maintain awareness of work areas; check barricades; do not approach operators; shout warnings to co-workers and bystanders. |
|  |  | Hearing loss | Wear ear plugs or ear muffs. |
|  | 4. Carrying saw | Cutting hands, feet, body | Idle engine when carrying less than 100 feet; stop engine when carrying saw more than 100 feet; never carry saw over shoulder. |
|  | Required Training: - Operation of chainsaw - First aid | Required Personal Protective Equipment (PPE) - Hard hat - Ear plugs - Safety glasses - Face shield - Gloves - Chaps - Work boots |  |