Chemical Storage Safety Checklist

DO:

- Check MSDSs before storing or removing a chemical.  Find out if it must be kept away from anything (light, water, air, high or low temperatures, or other chemicals) to prevent a dangerous reaction.

- Organize storage areas so chemicals don‘t have to be taken through areas containing incompatibles.

- Keep at least 18 inches between stored materials and sprinklers—36 inches if you’re storing very flammable material.

- Close containers as soon as you‘ve removed quantities you need.

- Wear the correct protective equipment when handling hazardous chemicals.  Check the MSDS for instructions.  Wear the correct gloves and safety goggles even if handling closed containers.

- Use bonding and grounding connections on flammable-liquid drums and small receptacles during transfer.

- Transport acid bottles in carriers, not by hand.

- Clean up even the smallest leaks promptly and properly.

- Check containers regularly for leaks or wear and report any problems immediately.

- Check containers regularly to see if any are outdated or never used.  Ask your supervisor if such chemicals can be disposed of properly.

- Report any container that doesn’t have a label.

- Keep packing materials such as straw or paper in a fire-resistant room equipped with a sprinkler system.

- Keep the work area clean, neat, and dust-free.

DON‘T:

- Take more from a hazardous chemical container than you need for the job.

- Use or remove anything from a container that doesn’t have a label.

- Store hazardous chemicals near heat or strong sunlight; they might 	expand and cause a fire or explosion.

- Stack materials so they block exits, firefighting equipment, alarms, or sprinklers.

- Smoke anywhere near the storage area.

- Siphon by mouth.

- Mix chemicals with each other or with any substance (even water!) without specific instructions to do so.

- Mix acids and water.

- Leave used flammable liquids containers near heat sources.