|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Operating a Drill Press |  |  |
|  | Task | Hazards | Controls |
|  | - Clean the table. | Eye injury from metal debris | Wear eye protection. Do not use compressed air. |
|  | 2. Load the vise. | Foot injury if the vise falls | Secure the vise on the table with T-pins. |
|  |  | Finger pinching while sliding the vise | Don’t let your fingers get under the vise unless you are lifting it from the table. Keep your eyes on the task. |
|  | 3. Lock the table in place. | Back strain | Don’t lean over the table to twist the lock handle. |
|  | 4. Load the bit. | Hand injury from the bit | Wear gloves. Don’t hold on the end of the bit. |
|  | 5. Start the drill. | None foreseen |  |
|  | 6. Feed the drill with the feed. | Injury caused by breaking the bit | Feed with the appropriate pressure. Use the appropriate bit for the type of metal. Wear eye protection. |
|  |  | Eye or skin damage from cutting oil | Use the lowest RPM. Wear eye protection. Wear a long sleeved shirt. |
|  |  | Hand injury from the exposed pulley near the feed handle | Make sure a pulley guard is in place. Don’t push the feed handle toward the pulley. |
|  | 7. Unload the vise. | Foot injury if the vise falls | Leave the vise secure on the table with T-pins until it is unloaded. |
|  |  | Finger pinching while sliding the vise | Don’t let your fingers get under the vise unless you’re lifting it from the table. Keep your eyes on the task. |
|  | 8. Clean the table. | Eye injury from metal debris | Wear eye protection. Do not use compressed air. |
|  | Required Training: - Operation of the drill press - First aid | Required Personal Protective Equipment (PPE) - Gloves - Eye protection |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | November 2002 |  |  |
| JSA Library Number: |  |  |  |