|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: |  | Task: | Hazardous Waste Drum Handling |  |
| --- | --- | --- | --- | --- |
|  |  | Name of Shop or Dept: |  |  |
|  |  | Job Title(s): |  |  |
|  |  | Analyzed by: |  |  |
|  |  | Date: |  |  |
| Required PPE: |  |  |  |  |
| Leather or similar gloves, safety glasses, steel toed shoes |  |  |  |  |
| Required/Recommended Trainings: |  |  |  |  |
| Use the ring clamp positioning and tightening clamp. Hazards associated with drum contents. |  |  |  |  |
| TASK | HAZARDS |  |  | CONTROLS |
| Examine the rim of the drum, lid, and sealing ring to be sure they will sit properly. | Pinch hand while handling parts Cuts or abrasions from burrs on metal part |  |  | Wear leather or similar gloves Use care (don’t grab) while examining parts Wear leather or similar gloves Keep your hands open (don’t grab) while examining parts |
| Place the lid on the drum, sit the ring, and tighten the bold using a wrench | Pinch hand while handling parts Cut abrasions from burrs on metal parts Abrasion or impact from tightening the bolt |  |  | See 1a-1 Use care and wear leather or similar gloves Position your body so the wrench can be easily turned |
| Attach the ring clamp for hard to sit rings, and torque into place by turning the handle | Impact and pinch while positioning the ring clamp Muscle strain from tightening the clamp |  |  | See 1a-1 Hold the clamp so the components don’t slip over the threaded shaft Position your body so the clamp tightening handle can be easily turned. |
| Secure and tighten the nut on the ring bold using the pneumatic hammer drill | Muscle strain from tightening the ring bolt |  |  | Seat the pneumatic hammer drill properly on the nut. Position your body to stabilize the position of the hammer drill prior to pulling the trigger |