|  | #### Job Safety Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Facilities Services |  |  |  |  |  |  |
|  | ## Maintenance of Fire Alarms, Panels, and Sensors |  |  |  |  |  |  |
|  | Task |  | Hazards |  | Controls |  |  |
|  | - Preparing work area |  | Injuries to passersby and bystanders |  |  | Isolate work area with barricades, caution tape, and/or on-site assistant |  |
|  |  |  | False alarm response; client anxiety/irritation/panic; disruption of campus business |  |  | Phone UCPD and notify building clients; schedule work appropriately |  |
|  | 2. Using ladders and lifts |  | Falls, slips, electrical hazards |  | Refer to ladder safety and aerial lift JSAs. |  |  |
|  | 3. Working in sub-ceilings, cramped spaces, custodial closets, machine rooms |  | Muscle strain, repetitive stress injuries |  | Stretching, frequent breaks, adequate hydration, alternate tasks with team partner |  |  |
|  |  |  | Heat stress |  | Frequent breaks, adequate hydration, alternate tasks with team partner |  |  |
|  |  |  | Head and/or eye injuries from falling or flying debris, dust inhalation |  | Protect head and eyes with hard hat and safety glasses; wear dust mask. |  |  |
|  |  |  | Hand and finger injuries from pinch points |  | Protect hands with gloves; avoid pinch points and moving machinery parts. |  |  |
|  |  |  | Skin/eye irritation from dirt and chemicals |  | Wear safety glasses and long-sleeved shirt. |  |  |
|  |  |  | Tripping hazards from cables and extension cords |  | Practice careful housekeeping; maintain awareness of location of equipment and tools. |  |  |
|  | 4. Repairing, replacing, or maintaining fire alarms, panels, or sensors | Electrical shock |  |  |  | Assess environment for water or damaged wiring/connectors before starting work; evaluate condition/age of building and consult written procedures (confidential to department) for that building before starting work. |  |
|  | 5. Testing horns | Hearing damage to self and others |  |  |  | Wear ear plugs; notify building clients and UCPD prior to tests. |  |
|  | Required Training: - Ladder safety - Aerial lift training - Electrical safety - Departmental building-specific alarm maintenance procedures |  |  | Required Personal Protective Equipment (PPE) - Safety glasses - Dust mask - Hard hat - Gloves - Ear plugs |  |  |  |
| Other Information: |  |  |  |  |  |  |  |
| Contributors: |  |  |  |  |  |  |  |
| Created: | July 2003 |  |  |  |  |  |  |
| JSA Library Number: |  |  |  |  |  |  |  |