# Eye and Face Protection

Employees can be exposed to a large number of hazards that pose danger to their eyes and face. OSHA requires employers to ensure that employees have appropriate eye or face protection if they are exposed to eye or face hazards from flying particles, molten metal, liquid chemicals, acids or caustic liquids, chemical gases or vapors, potentially infected material or potentially harmful light radiation.

Many occupational eye injuries occur because workers are not wearing any eye protection while others result from wearing improper or poorly fitting eye protection.  Employers must be sure that their employees wear appropriate eye and face protection and that the selected form of protection is appropriate to the work being performed and properly fits each worker exposed to the hazard.

## Prescription Lenses

Everyday use of prescription corrective lenses will not provide adequate protection against most occupational eye and face hazards, so employers must make sure that employees with corrective lenses either wear eye protection that incorporates the prescription into the design or wear additional eye protection over their prescription lenses.  It is important to ensure that the protective eyewear does not disturb the proper positioning of the prescription lenses so that the employee's vision will not be inhibited or limited.  Also, employees who wear contact lenses must wear eye or face PPE when working in hazardous conditions.

## Eye Protection

Examples of potential eye or face injuries include:

- Dust, dirt, metal or wood chips entering the eye from activities such as chipping, grinding, sawing, hammering, the use of power tools or even strong wind forces.

- Chemical splashes from corrosive substances, hot liquids, solvents or other hazardous solutions.

- Objects swinging into the eye or face, such as tree limbs, chains, tools or ropes.

- Radiant energy from welding, harmful rays from the use of lasers or other radiant light (as well as heat, glare, sparks, splash and flying particles).

## Types of Eye Protection

Selecting the most suitable eye and face protection for employees should take into consideration the following elements:

- Ability to protect against specific workplace hazards.

- Should fit properly and be reasonably comfortable to wear.

- Should provide unrestricted vision and movement.

- Should be durable and cleanable.

- Should allow unrestricted functioning of any other required PPE.

The eye and face protection selected for employee use must clearly identify the manufacturer.

An employer may choose to provide one pair of protective eyewear for each position rather than individual eyewear for each employee.  If this is done, the employer must make sure that employees disinfect shared protective eyewear after each use.  Protective eyewear with corrective lenses may only be used by the employee for whom the corrective prescription was issued and may not be shared among employees.

Some of the most common types of eye and face protection include the following:

- Safety spectacles.  These protective eyeglasses have safety frames constructed of metal or plastic and impact-resistant lenses.  Side shields are available on some models.

- Goggles.  These are tight-fitting eye protection that completely cover the eyes, eye sockets and the facial area immediately surrounding the eyes and provide protection from impact, dust and splashes.  Some goggles will fit over corrective lenses.

- Welding shields.  Constructed of vulcanized fiber or fiberglass and fitted with a filtered lens, welding shields protect eyes from burns caused by infrared or intense radiant light; they also protect both the eyes and face from flying sparks, metal spatter and slag chips produced during welding, brazing, soldering and cutting operations.  OSHA requires filter lenses to have a shade number appropriate to protect against the specific hazards of the work being performed in order to protect against harmful light radiation.

- Laser safety goggles.  These specialty goggles protect against intense concentrations of light produced by lasers.  The type of laser safety goggles an employer chooses will depend upon the equipment and operating conditions in the workplace.

- Face shields.  These transparent sheets of plastic extend from the eyebrows to below the chin and across the entire width of the employee's head.  Some are polarized for glare protection. Face shields protect against nuisance dusts and potential splashes or sprays of hazardous liquids but will not provide adequate protection against impact hazards.  Face shields used in combination with goggles or safety spectacles will provide additional protection against impact hazards.

Each type of protective eyewear is designed to protect against specific hazards.  Employers can identify the specific workplace hazards that threaten employees' eyes and faces by completing a hazard assessment.