JOB SAFETY ANALYSIS (JSA)

| Job: Operating Forklift | Dept.: Road & Bridge | Date: 9.24.10 |
| --- | --- | --- |

| Supervisor: Joe Carter | JSA No. #6 New  Revised  | Analysis Performed By: Road & Bridge Employees & Safety Coordinator |
| --- | --- | --- |
| Approved by: Joe Carter | Operator: Operators & Laborers | Note: Danger: Tipping forklift over - Always wear seatbelt, maintain stability triangle and proper center of gravity at all times. |

Required/Recommended Personal Protective Equipment (PPE): Safety Vest, safety-toed boots should be worn at all times. Appropriate gloves should be worn as needed. Hard hat protection required as needed. Notes - *While working outdoors dehydration is always a problem - drink adequate amounts of water. *Wear appropriate clothing for weather conditions *When working outside whatever the season sun screen is advised

| Item | Sequence of Basic Job Steps | Potential Incident/Hazard | Recommended Safe Job Procedure |
| --- | --- | --- | --- |
| 1. | Pre-operation Safety Check | *Untrained operator | *Training on forklift design, controls and instrumentation. *Training on the forklift stability and the proper way to transport, load, and stack on the forklift. |
| 2. | Walk around inspection: tires, lights, frame (stress), leaks, fluid levels, mirrors | *Spills/oil & fuel on hands & clothing *Pinch points *Slip, trip & fall *Back strain | *Wear protective clothing (coveralls) or have change of clothes available *Wear appropriate gloves *Awareness/condition of work area, obstacles *3 point contact *Proper bending & lifting technique |
| 3. | Vehicle startup: check gauges, oil & air pressure, hydraulics, and brakes | *Slip, trip & fall climbing into vehicle | *Maintain 3 point contact * Check for ice build up if working outside in cold months |

| 4 | Cranking & Driving | *Rolling the wheels off an edge, ramps and loading docks. | *Buckle your seat belt when you’re in the truck. *Stay well back from any edge. *Never turn around on the slope. *Drive with your load on the uphill side. This means you drive up slopes with your load in front and drive down slopes in reverse. |
| --- | --- | --- | --- |
| 5. | Operating the forks | *Running into obstacles | *Slow down and sound your horn at corners, intersections and places with limited view. *Slow down for turns. *Drive in reverse if your vision is obstructed. *Make sure that you have enough overhead clearance. |
| 6. | Loading | *Load falling off forklift. *Damage to material by loading improperly. *Overloading *Pinching hands between the truck and other objects. * | *Spread forks as wide as possible for transporting the load. *Tilt the mast slightly forward and align the forks with the base of the load. *Proceed slowly and insert the forks into the base of the load. Ensure that at least 2/3rd the length of the fork enter the base of the load. *Lift forks approximately 8 inches for the load to clear the floor. *Tilt mast back in preparation for traveling. |
| 7. | Transporting the load | *Struck against - Running into objects. *Struck by - Collisions with other equipment. | *BE ALERT *Wear gloves to protect your hands. *Keep your arms and hands in the truck. *Keep the controls and your hands clean and dry. |
| 8. | Unloading | *Injury to other personnel *Damage to other equipment | *Position the truck at the desired location. *If stacking loads, adjust the height of the load as appropriate. *Tilt the mast forward to place the forks parallel to the ground. *Once the load is set, slowly back away from the load. *When clear of the load, place the forks at the proper height for traveling. |
| 9. | Parking the Forklift | *Running over and crushing the operator. *Slip, trip, and fall | *Never leave the truck until you lower the lifting mechanism, put controls in neutral and set the brakes. Turn the truck off. *Always administer the emergency brake. *Leave the truck- Keeping your balance by maintaining a 3 point stance. *Be aware of condition of walking surface. |