This template is a Microsoft Word table – use MS Word’s online “Help” feature for instructions about how to add text and format tables.

|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Environment, Health & Safety– Radioactive Waste |  |  |
|  | ## Handling Lead Waste |  |  |
|  | Task | Hazards | Controls |
|  | - Load lead bricks, pigs, or sheets onto a cart, dolly, or hand truck. | Foot injury from dropping the lead. | Wear appropriate PPE. Stack the lead securely. |
|  |  | Hand pinching. | Wear thick gloves over disposable gloves. Pick up the lead from the top while moving it. |
|  |  | Muscle strain. | Move the lead using proper lifting techniques. |
|  |  | Pb exposure from disturbing the surfaces during movement. | See 1a-1. |
|  | 2. Wheel the cart/dolly/hand-truck to the area where bricks will be stored or monitored. | Foot injury from the loaded cart. | See 1a-1. Be careful to keep your feet out of the path of the cart. |
|  |  | Muscle strain. | Move the cart/dolly/hand truck using proper body mechanics. |
|  | 3. Unload the bricks from the cart. | Foot or hand injury from dropping the lead. | See 1a-1. See 1a-2. |
|  |  | Hand pinching. | See 1b-1. See 1b-2. |
|  |  | Muscle strain. | See 1c-1. |
|  |  | Pb exposure from disturbing the surfaces during movement. | See 1a-1. |
|  | 4. Stacking lead in place. | Foot injury from dropping the lead. | See 1a-1. Pick up the lead from the bottom while moving it. |
|  |  | Hand Pinching. | See 1b-1. See 1b-2. |
|  |  | Muscle strain. | See 1a-1. |
|  |  | Pb exposure from disturbing the surfaces during movement. | See 1a-1. |
|  | 5. Swipe and monitor lead for radioactive contamination. | Foot or hand injury from dropping the lead. | See 1a-1. See 4a-2. |
|  |  | Hand pinching. | See 1a-1. See 1b-2. |
|  |  | Muscle strain. | Move your body in such a way as not to cause strain. Carefully position lead for swiping and monitoring. |
|  |  | Pb Exposure from disturbing the surface of the lead. | See 1a-1. |
|  | 6. Load lead onto the cart/dolly/hand-truck and return to storage. | Foot from dropping the lead. | See 1a-1. See 1a-2. |
|  |  | Hand pinching. | See 1a-1. See 1b-2. |
|  |  | Muscle strain. | See 1c-1. |
|  |  | Pb Exposure from disturbing the surface during movement. | See 1a-1. |
|  | 7. Wheel cart to the lead storage area. | Foot injury from the loaded cart. | See 1a-1. See 2a-2. |
|  |  | Muscle strain. | See 2b-1. |
|  | 8. Unload lead from the cart/dolly/hand truck and move it to the storage area. | Foot injury from dropping the lead. | See 1a-1. See 4a-2. See 1a-2. |
|  |  | Hand pinching. | See 1a-1. See 1b-2. |
|  |  | Muscle strain. | See 1c-1. |
|  |  | Pb Exposure. | See 1a-1. |
|  | Required Training: 1. Proper lifting techniques. 2. Cal/OSHA Lead awareness training. 3. Radiation Safety Training. 4. RP-20, "Handling and Disposal of Radioactive Waste." | Required Personal Protective Equipment (PPE): Disposable gloves, leather gloves, steel-toed shoes, safety glasses, respiratory protection |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | June 2005 |  |  |
| JSA Library Number: |  |  |  |