|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: | Task: |  | Municipal trash |
| --- | --- | --- | --- |
|  | Name of Shop or Dept: |  | OCH-Rooms |
|  | Job Title(s): |  | Custodian, Assistant & Senior |
|  | Analyzed by: |  | M. Williams |
|  | Date: |  | 05/09/13 |
| Required PPE: |  |  |  |
| Non-Slip shoes Neoprene (thick) gloves |  |  |  |
| Required/Recommended Trainings: |  |  |  |
| STF Safety Ergonomics |  |  |  |
| TASK | HAZARDS | CONTROLS |  |
| Manual trash handling | Lacerations Punctures Slips Trips Falls Sprains Strains | - Don PPE, including non-slip shoes and gloves prior to handling trash. - Visually look over the contents of the trash to make sure nothing is protruding from the can that can cause injury - Check the weight of the trash can by tilting or tapping it, if the bag is too heavy avoid lifting straight up. - For heavy loads tilt the trash can down to eliminate the need to pull trash vertically. - Use proper lifting techniques. Bend your knees, keep close to the waste can and keep your back straight as you pick up or lower trash cans. - Remain upright while relining the trash cans, avoid bending at the back. - Whenever possible, use both hands to pick up and lower trash cans. - For small office trash cans one hand can be used, but alternate hands whenever possible, especially when emptying a high frequency of offices. - Never manually push the trash down with your hands as there may be sharp objects. - Remember to think before you lift and focus on the specific task. |  |