# POWERED HAND TOOLS – Part 1

When and how should you inspect powered hand tools?

- Inspect tools for any damage prior to each use.

- Check the handle and body casing of the tool for cracks or other damage.

- If the tool has auxiliary or double handles, check to see that they installed securely.

- Inspect cords for defects: check the power cord for cracking, fraying, and other signs of wear or faults in the cord insulation.

- Check for damaged switches and ones with faulty trigger locks.

- Inspect the plug for cracks and for missing, loose or faulty prongs.

What should you do if you find a tool defective?

- If a tool is defective, remove it from service, and tag it clearly "Out of service for repair".

- Replace damaged equipment immediately - do not use defective tools "temporarily".

- Have tools repaired by a qualified person - do not attempt field repairs.