|  | Injury & Illness Prevention Program Job Safety Analysis Form |
| --- | --- |

| Picture of task/equipment: | Task: |  | Weed Trimmer |
| --- | --- | --- | --- |
|  | Name of Shop or Dept: |  | OCH-Grounds, UA & LACC |
|  | Job Title(s): |  | Groundskeepers |
|  | Analyzed by: |  | M. Williams |
|  | Date: |  | 03/07/13 |
| Required PPE: |  |  |  |
| Work boots Gloves hearing protection safety glasses dust mask Safety vest |  |  |  |
| Required/Recommended Trainings: |  |  |  |
| STF Safety Equipment safety precautions |  |  |  |
| TASK | HAZARDS | CONTROLS |  |
| Operating a gas powered weed trimmer | Struck By’s Lacerations Slips Trips Falls Vapor inhalation Fire | - Don PPE before starting equipment, including non-slip work boots, Gloves, Hearing Protection, Safety glasses, a mask and a safety vest (when working near public roadways). - Wear long pants, as flying debris can come in contact with lower limbs. - Make sure all guards are in place before operating equipment. - Inspect spark plug cap and lead to make sure they are not damaged, otherwise electric shock can occur. - Do not operate within 50ft of persons or traffic as injury or ricochet can occur. - Do not operate next to glass, such as windows. - Watch for uneven surfaces, holes and slopes; take extra care with footing to avoid STF’s. - Keep hands, arms and dry debris away from the muffler, it can result in burns or fires. - Never start the equipment indoors as exhaust can create a harmful breathing atmosphere. - Hold trimmer head just above ground and let the cord cut; never force the cutter into grass or weeds. - Do not try to make adjustments to the cutting cord while the machine is on. - Do not try to remove debris while the machine is in operation; make sure machine is off before clearing. |  |