|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Facilities Services |  |  |
|  | ## Using an Oxy-Acetylene Welder |  |  |
|  | ## * A welding area of a minimum of 35 feet in diameter must be established. Flammable materials must be removed from the welding area. A fire extinguisher with a minimum rating of 2A20B:C must be readily available to the welder. A fire alarm must be readily available. Ensure that ventilation is sufficient to avoid setting off a smoke-detector fire alarm. |  |  |
|  | Task | Hazards | Controls |
|  | - Close off welding area. | Flashing | Close welding curtain to shield outsiders from flashing. |
|  | 2. Prepare for welding. | Inhalation of fumes | Turn on exhaust fan and timer. |
|  |  | Flashing | Wear welding hood. |
|  |  | Sparks | Wear welding jacket, apron, gloves, work shoes. |
|  |  | Slag splatter | Wear welding jacket, apron, gloves, work shoes. |
|  | 3. Set gauges, turn on gas cylinders. | Pinching fingers and hands | Avoid pinch points |
|  | 4. Clean tip, make sure hose valves are tight, unwrap hoses. | Pinching fingers | Avoid pinch points. |
|  |  | Tripping | Keep hoses untangled and free from feet. |
|  | 5. Use striker to light torch | Burning fingers, hands body; flashing | Wear welding hood, welding jacket, apron, gloves, work shoes. |
|  | 6. Apply flame to material. | Flashing, sparks, slag splatter | Wear welding hood, welding jacket, apron, gloves, work shoes. |
|  | 7. Allow material to cool on workbench. | Burn to hands or fingers | Wear gloves. Chalk mark welded area “Hot.” |
|  | 8. Close valves, bleed off regulators, wrap hoses. | Pinching fingers | Avoid pinch points. |
|  |  | Tripping | Keep hoses untangled and free from feet. |
|  | 9. Use chipping hammer to remove excess slag. | Eye damage by flying debris from hammer strikes | Wear safety glasses. |
|  |  | Injuring fingers with hammer | Use caution to avoid striking fingers or hands with hammer. |
|  | Required Training: - Operation of gas welder - Operation of a fire extinguisher - Location and use of the fire alarm | Required Personal Protective Equipment (PPE) - Welding hood - Welding jacket and apron - Gloves - Safety glasses - Work shoes |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | April 2003 |  |  |
| JSA Library Number: |  |  |  |