|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | ## Operating a Table Saw |  |  |
|  | Task | Hazards | Controls |
|  | - Position feeder out of the way for blade and fence adjustments. | Injury from blade while not in operation. Possible accidental starting. | Be aware of the switch and blade at all times. |
|  | 2. Adjust fence. | Pinching/cutting fingers from fence adjustment. | Don’t lift the fence during adjustment. Keep fingers out of moving parts. |
|  | 3. Verify blade-to-fence distance with a measuring tape. | Cut hand. | Keep fingers away from the blade. |
|  | 4. Adjust height and angle of blade. | Cut hand. | Keep fingers away from the blade. Suggest blade be 1/8” above stock. |
|  | 5. Start the saw. | Projectiles for rotating blade. | Verify that there is no wood or other debris around the blade. |
|  |  | Stock kickback. | Verify that the fence is parallel to the blade. Verify that the feeder has enough downward force to stop kickback. Don’t watch the blade from the side of the saw. |
|  |  | Eye injury. | Wear safety goggles or face shield. |
|  |  | Hearing damage. | Wear earplugs or earmuffs. |
|  | Required Training: Operator must receive one hour of supervised training on the saw before operating the saw independently. | Required Personal Protective Equipment (PPE): Hearing and eye protection—goggles, shield, earplugs. |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | April 2005 |  |  |
| JSA Library Number: |  |  |  |