.

|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | ## Sewing Machine Operation for Insulators |  |  |
|  | Task | Hazards | Controls |
|  | - Wind Bobbin | Belt Pinch | Keep hand back from the belt while engaging friction drive on the belt |
|  | 2. Tilt back machine to mount bobbin | Hand impact and pinching | Insert block to prevent machine from falling on your hand. |
|  | 3. Thread the thread | No known hazards |  |
|  | 4. Thread the needle | Needle penetration of finger | Keep your foot off the accelerator and do not move the machine with your hand |
|  | 5. Operate the machine and feed the fabric | Cut from insulation pad fabric | Keep hands away from fabric edge while feeding the fabric |
|  |  | Needle penetration of finger | Keep hands away from needle during operation |
|  |  | Eye and face injury if needle breaks | Keep face back from needle during operation and wear safety glasses |
|  |  | Pin penetration in the mouth or choking from pin inhalation | Do not hold pins in your mouth |
|  |  | Pin penetration in the hands arm or butt. | Keep the pins from surfaces such as the sewing table or chairs. Use a magnet or a pin cushion to hold the pins |
|  | 6. Operating the machine by hand for walkback or backstitch | Belt pinch | Keep your foot off the accelerator |
|  | 7. Chair adjustment | Strained muscles | Keep the chair and lighting adjusted so you can sit up straight and easily access and see the operation |
|  | 8. |  |  |
|  | Required Training: | Required Personal Protective Equipment (PPE) Safety Glasses |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | December 2008 |  |  |