.

|  | #### Job Safety Analysis |  |  |
| --- | --- | --- | --- |
|  | Environment, Health & Safety – Radioactive Waste |  |  |
|  | ## Segregating Waste |  |  |
|  | Task | Hazards | Controls |
|  | - Opening the drum or container. | Abrasions from the drum and muscle strains. | See the JSAs on opening and closing the drums. |
|  | 2. Segregating the containers according to the isotope and waste categories. | Cuts from sharps such as broken glass containers. | Wear gloves. Lift from the corner of the bag and examine for breakage and other sharps. |
|  |  | Radiation exposure due to leaking bags. | Lift from the corner of the bag from the opening edge and be careful not to spill contents. |
|  | 3. Weekly monitoring of radioactivity at each drum. | Potential radiation exposure due to unforeseen release of materials. | Use the radiation detector following procedures associated with use of the monitor. |
|  | Required Training: - RP-20 Handling and Disposal of Radioactive Waste. - RWI-2 Monitoring and Surveys. - RWI-5: Decay Processing. - RWI-9: Waste Pick-Up Procedure. - RW1-10: Discrepant Waste Handling. - RWI-60: Waste Characterization. - Spill Response Procedures. | Required Personal Protective Equipment (PPE): Dosimetry, lab coat, safety glasses, steel-toed shoes |  |
| Other Information: |  |  |  |
| Contributors: |  |  |  |
| Created: | June 2005 |  |  |
| JSA Library Number: |  |  |  |